Temperature & Humidity Sensor

The ESP8266 microcontroller was programmed using the Arduino IDE to collect both temperature and humidity data from a DHT11 sensor. The device connects to a WiFi network and publishes sensor readings to a public MQTT broker hosted at 'test.mosquitto.org'. Every 3 seconds, the DHT11 sensor measures temperature and humidity. Each value is compared against predefined threshold levels and classified into status categories:

Temperature thresholds:
- Safe: ≤ 15°C
- Caution: 15°C – 20°C
- Warning: > 20°C

Humidity thresholds:
- Safe: ≤ 60%
- Caution: 60% – 70%
- Warning: > 70%

The ESP8266 publishes two separate JSON messages to MQTT topics:
- Temperature → devices/NAPIoT-P2/anelieb
- Humidity → devices/NAPIoT-P2/anelieb12

Example temperature payload:
{"temperature": 22.5, "status": "Warning"}
Example humidity payload:
{"humidity": 68.0, "status": "Caution"}

FlowFuse receives the JSON data using MQTT input nodes and passes them through JSON decoder nodes. Each decoded payload is handled by a function node that splits the message into two outputs:
- The numeric value to a gauge
- A status string to a notification or text widget

A second function node processes the `status` field and generates a formatted message based on the threshold level. This design ensures clear, user-friendly monitoring of both temperature and humidity values on the dashboard.

The final dashboard contains the following widgets:
- A gauge for real-time temperature readings
- A gauge for real-time humidity readings
- A text and notification panel for each to alert the user

Temperature:

Humidity: